Engineering Team:

Matthew Belden

Cynthia Lopez

Matthew Garcia

Cesar Elizondo

Louis Tiger

Arash Ebad

Kristin Makowski

Mark Parangue

Chad Higgins

Thomas Gerstenberg

Robert Paulino

Darren George

Sponsored By:

San Diego Gas & Electric

Submitted To:

John Kennedy and Dr. Lal Tummala

Table of Contents:

Introduction:

- Abstract..........................................................................................................3

- Project Description.........................................................................................3

Design:

- Block Diagram (Hardware) .............................................................................4

- Illustrations (Hardware and Software) ..........................................................5

- Block Diagram (Software) ..............................................................................6

- Performance Requirements (Hardware and Software) .................................8

Testing and Verification:

- Testing Procedures.........................................................................................9

- Benchmarks..................................................................................................10

Project Management:

- Project Plan..................................................................................................11

- Milestones....................................................................................................12

Budget:

- Cost Analysis.................................................................................................21

Other:

- Flyer..............................................................................................................23

INTRODUCTION

Abstract:

Energy prices and use seem to be rising on an annual basis that makes people worry about where their money is going.  This past Fall's rate increase in some areas went up as high as 30 percent.  How can San Diego Residents save money as their energy demands increase from the use of air conditioners, appliances, and POOLS?

With a Variable Frequency Drive (VFD) in use with a pool pump, we plan to prove that VFDs are worth investing in.  A VFD is designed to control a motor by taking in an alternating current (AC) voltage and altering the output frequency and AC voltage to the device. This allows a higher level of control over the amount of energy being used by the pump.

Project Description:

There is not a lot of proof in testing VFDs for reliability and sustainability, but we plan to thoroughly test the use of VFDs to distinguish the facts from the fiction.  There are many videos online that argue for and against the use of VFDs, but our demonstration will help determine the validity of these controllers.  Our demonstration will consist of a water-circulating pump system to facilitate as a smaller version of a pool.  The system will consist of the Energy Wise Outlet (EWO) connected to a VFD, which will regulate the pump that is circulating water.  Three sensors along with the EWO will be monitoring the dynamics of the system and a microcontroller will send the data wirelessly to a backend server, which will host a web application for data monitoring and user control.  In addition, there will be a touch-screen interface for user control on the system for quick access to controls.

The EWO is a project that was completed last semester, Fall 2013, and will be utilized for its purpose to provide following information: line voltage, line current, line frequency, power factor, real power (kW), apparent power (kVA), and reactive power.  We will use a single phase water pump to simulate as a pool pump.  Clear PVC was chosen for our piping material so that customers can see the flow of water through our system.  The VFD is a single phase input and output controller for the pump.  Three sensors will be installed via hardwire: two sensors for pressure and flow of the water line, and a sensor for the temperature of the pump.  There will be an Arduino microcontroller to collect the sensor data and control the VFD and will communicate the data over Zigbee wireless protocol (802.15.4) to the Raspberry Pi.  The Raspberry Pi will control the touch-screen interface as well as act as the gateway to the OSI PI server and web application.  The Raspberry Pi will communicate with the server over the wireless 802.11 standard.  The OSI Pi server will store all of the data points collected by the sensors as well as host a cross-platform web application that will allow users to view data and interact with the VFD controls.  Finally, the electrical panel will be a clear casing that will allow potential buyers and enthusiasts to see the different components and realistic view of the materials used.

With our design, we plan to provide a user friendly interface to control the VFD via the web, accessed through a personal computer, tablet, or smart phone.  As potential customers may see, adjusting the VFD can potentially save money as their pump will not have to be on full power continuously throughout the day wasting energy.  By displaying the circulating water, we can visually demonstrate the effects of different flow rates to show adjustments to pressure, pump reaction, and how the VFD can be used to decrease the chance of equipment failure.

Design

Block Diagram and Mock-Up Illustrations:

-Hardware Block and Tank Diagram

-Hardware Electrical Panel

-Software Block Diagrams

-Software User Interface (UI) Mockups

Performance Requirement:

-Hardware

- General Specifications

  - Arduino Microcontroller must be able to communicate with VFD and Raspberry Pi.  This will also handle the commands from the Raspberry Pi and collect feedback from the different sensors.

  - The VFD must show improved electrical and mechanical efficiency over a set time period.  The data will be shown through analytical and graphical data.

    - Electrical efficiency is the measured performance divided by total power output.

    - Mechanical efficiency is the measured performance divided by the ideal performance.

  - All three sensors must be able to transmit data to the microcontroller.  Initially hard wired to the Arduino, and transmitted via Zigbee for wireless links.

  - Water pump will have to circulate 5 gallons of water at a certain flow rate.

  - The tank will be sized for 10 gallons (scale of 1:1,615 of average swimming pool).

  - Switching unit will be an electrical relay that must be able to switch and delay from the VFD to 120VAC directly for pump operation without the possibility of both outputs being on at the same time.

-Software

- General Specifications

  - Software shall facilitate the data transfer between the web app, VFD, and PI database

    - Web based user interface shall

      - Facilitate cross platform use (android, apple, windows)

      - Allow user control of the VFD

        - Start/Stop, Set runtime, adjust speed/frequency of motor

      - Give a visual display of data from sensors (voltage, current, power factor, flow, pressure, rpm)

      - Calculate and display energy efficiency and money saved over certain timespans

    - The Touch screen interface shall

      - Display and control the current network settings

      - Give basic controls for the VFD and display its current settings

  - Communication Devices

    - The Raspberry Pi (microcontroller) shall

      - Send commands to Arduino to adjust VFD

      - Receive and process sensor data from the Arduino and send it to the OSI PI database through WIFI (IEEE 802.11 standard)

      - Drive a touch screen tablet to control the VFD and network settings

      - Have the ability to run off a 5 volt battery source for xx hours

    - The Xbee ZB (RF modules) shall

      - Communicate data sent between Arduino and Raspberry Pi

      - Operate using Zigbee protocol (IEEE 802.15.4 standard)

    - The Lenovo Laptop(server) shall

      - Contain and manage the OSISoft PI time-series database

      - Maintain the data collected by sensors and current VFD settings

      - Host the web application for the client

Testing and Verification

Testing Procedures:

The V.I.C.E. system will be tested initially as individual sub-systems and then as one integrated system in order to verify operation.

- Hardware:

- EWO will be tested against known factors, such as a shunt resistor, in order to determine accuracy.

- Flow and pressure sensors will have a voltage supplied, and while using a digital multi-meter and oscilloscope, will obtain functionality while varying the flow of the system.

- Temperature sensor will be verified using a pre-calibrated temperature sensor.

- VFD and Motor will be verified using the flow, pressure, and temperature sensors to verify operation while varying frequency.

- Software:

- ARM Cortez-M3 (Arduino) microcontroller will be programmed to obtain sensor and VFD data, which will be packetized and sent to the Raspberry Pi.

- ARM1176JZF-S (Raspberry Pi) will be programmed to establish connection between the Arduino, OSI Pi server, and user interface.  This will respond to users inputs by changing the required setting on the microcontroller which will control the VFD.

- Zigbee modules will be verified by determining if the data packets are available.

After each sub-system is functioning properly, we can begin integrating the different systems.  Integration testing will be the most challenging aspect of our design from one day to the next.  These systems will not just be tested for functional operation, but also for durability.  After we fix the different issues that may arise during testing, we will be able to demonstrate a properly operational circulating system that will provide important data for the customer.

Benchmarks:

-Hardware

The following benchmarks have been placed in order to ensure satisfactory operation.

1) Pump/VFD- Variable Frequency Drive will have a noticeable effect on the pump.

2) Sensors- Collecting correct data throughout the system.

3) Relay- Switch the power being supplied to the pump from the VFD to GFCI.

4) Microcontroller (ARM Cortex-M3)- Interpret data from sensors and variable frequency drive, and will also transmit data to the Raspberry Pi.  Will have ability to change settings on the VFD and control the relay switching unit.

5) Microcontroller/Raspberry Pi- Wireless connection between the ARM Cortex-M3 and Arm1176JZF-S processors using the Xbee (Zigbee) modules.

-Software

1) Successful communication between Server and Web Application using dummy values.

2) Successful communication between the Raspberry Pi and Server using dummy values.

3) Ability to update the Web Application in real time from dummy values created by the Raspberry Pi.

4) Successful request/send communication between the Arduino and Raspberry Pi over Zigbee.

5) Successful request/send communication from the Server to the Arduino.

6) Touchscreen GUI developed and tested.

7) Ability to update the Web Application in real time from real values sent from Arduino.

8) Ability to control the VFD through the Web Application.

9) Ability to control the VFD through the Touchscreen GUI.

Project Management

Project Plan (Hardware and Software)

The project plan consists of distributing the resources allotted for the project to ensure milestones are met and all members are being utilized.  Tasks are distributed weekly and mainly consist of research.  Along with weekly tasks members have been distributed milestone tasks.  These tasks are assigned in addition to the weekly tasks to allow each member to have accountability for a portion of the project and ensure we have an expert for each area of the project.  Each member of the team has also been assigned a job.  Each job is crucial for the team to successfully complete the project on time. The milestones are displayed in the Gantt chart along with the weekly tasks and deadlines for the semester.  We have milestones to complete each week which will keep us on track to complete the main milestones listed below.

Milestones (Gantt Chart)

-Hardware

-Hardware (cont'd)

-Hardware (cont'd)

-Hardware (cont'd)

-Software

-Software (Cont'd)

-Software (Cont'd)

-Software (Cont'd)

-Software (Cont'd)

Budget

| Software |  |
| --- | --- |
| Raspberry PI Microcontrollers and Initial Kit | $180.00 |
| XB Nodes/ Adapter Kits | $150.00 |
| Raspberry PI Touchscreen | $50.00 |
| Self-contained PI Casing | $25.00 |
| Wireless Router | $50.00 |
| Hardware |  |
| VFD | $400.00 |
| Water Pump | $100.00 |
| Acrylic Tank | $100.00 |
| Plumbing/Valve | $100.00 |
| Relay | $50.00 |
| Rs485-TTL Converter | $20.00 |
| Sensors (Pressure, Flow, Temp) | $300.00 |
| Electrical Box | $100.00 |
| Dc-Dc Converter | $50.00 |
| Arduino Microcontroller | $50.00 |
| Mounting Hardware | $300.00 |
| Total Costs |  |
| $2,025.00 |  |

The supplied list gives a view of our projected costs throughout the project. This total, $2025.00, doesn't include the smart outlet we plan to use, nor does it include shipping. As a team, we feel the a $2200.00 budget can be achieved and will allow us to have a great finished product.

Promotional Flyer